TwistingLogger Quick Reference

Pi 4 → Pi 5 migration and on-demand CSV export

System addresses: TwistingLogger 192.168.20.141  |  TW2 PLC 192.168.20.10

# 1. Move the existing microSD card to the Pi 5

While still connected to the Pi 4, stop the logger and shut down cleanly:

sudo systemctl stop twistinglogger sudo shutdown -h now

Wait until the Pi is fully off. Move the microSD card to the Pi 5, connect Ethernet to the controls network, and then apply power.

# 2. Connect from Windows and verify operation

ssh twistinglogger@192.168.20.141

After signing in, run each verification command separately:

date hostname -I systemctl is-active twistinglogger systemctl is-active twistinglogger-export.timer cat /opt/twistinglogger/status.json

Expected: both services report active. In status.json, each live group should show connected: true, quality: good, and sequence values that increase over time.

# 3. Export today early (partial-day snapshot)

Run these commands on the Pi. The force option permits an export before the day is complete:

mkdir -p /opt/twistinglogger/exports/Early /opt/twistinglogger/venv/bin/python /opt/twistinglogger/export_zip.py --day "$(date +%F)" --output-dir /opt/twistinglogger/exports/Early --force

# 4. Export a specific date early

Replace the example date with the required YYYY-MM-DD date:

/opt/twistinglogger/venv/bin/python /opt/twistinglogger/export_zip.py --day "2026-09-16" --output-dir /opt/twistinglogger/exports/Early --force

# 5. Open the export folder from Windows

Paste this path into File Explorer or the Windows Run dialog:

\\192.168.20.141\TwistingLogger\Early

Important: An early ZIP is only a snapshot through the export time. The normal completed-day ZIP is still generated automatically in the main TwistingLogger share after midnight.